"3D interpreter of work drawings" program

Table of contents

1. Introduction.........................................  2

2. Interpreted objects (IO).............................  3

2. 1. IO Plan........................................  3

2. 2. IO Section.....................................  3

2. 3. IO Coordination axis for marking X axis........  4

2. 4. IO Coordination axis for marking Y axis........  4

2. 5. IO Coordination axis for marking Z axis........  5

2. 6. IO Section line................................  5

2. 7. IO Local Section line..........................  6

2. 8. IO Wall........................................  7

2. 9. IO Front wall..................................  8

2.10. IO Front wall opening..........................  9

2.11. IO Slab........................................ 10

2.12. IO Slab opening................................ 12

2.13. IO Ramp........................................ 12

2.14. IO Column...................................... 14

2.15. IO Column opening.............................. 15

2.16. IO Beam........................................ 16

2.17. IO Rafter...................................... 17

3. Story multiplication................................. 20

4. Step-by-step object detailing........................ 20

5. Getting Started...................................... 21

6. Program installation and start....................... 22

1. Introduction

"3d-int" program builds a 3D model of building on the basis of formalized construction drawings.

To make the drawing interpreted it is necessary to create a set of layers with predefined names and use predefined graphic primitives as drawing tools.

Plans and sections on drawing are principal schemes explaining how building is arranged. Along with its advantages, presentation of information via principal schemes has a disadvantage - probability of mismatch between the schemes located in different parts of the drawing or on different drawings of project. Mismatches result in slowdown and increase in the cost of construction.

Electronic automatic assembly of a spatial model of building prepared on the basis of drawings allows detection of errors at the design stage.

The purpose of the current version of 3D interpreter is check of correctness (consistency) of geometrical information presented on drawings.

2. Interpreted objects (IO)

2.1. IO Plan

Layer name:

"Plans"

IO elements:

1. Frame

Closed polyline (rectangle).

2. Plan title

Single-line text, e.g. "Plan at EL. 0.000".

Significant information in presented text is the number 0.000

(plan elevation mark).

Explanations

1. The Plan title is located randomly inside frame.

2. Inside the plan frame other interpreted objects are located: coordination axes, section lines, projections of the building structural elements.

3. The structural elements get X and Y coordinates according to arrangement of their projections on building plans.

4. On the basis of plans the story-by-story building structuring is performed.

The interpreter checks all plans of the drawing (dwg file) and builds an ordered list of stories according to plan elevation marks. Story height is calculated as difference in elevations of this and superjacent story from the ordered list. Height of the uppermost story is taken unlimited.

2.2. IO Section

Layer name:

"Sections"

IO elements:

1. Frame

Closed polyline (rectangle).

2. Section title

Single-line text, e.g. "Section 1-1".

Significant information in presented text is the number 1

(section designation).

Explanations

1. The title is located randomly inside frame.

2. Inside the section frame other interpreted objects are located: coordination axes, projections of the building structural elements.

3. On the basis of sections Z coordinates of the building structural elements are defined.

2.3. IO Coordination axis for marking X axis

Layer name:

"Axis for X"

IO elements:

1. Axis

Vertical line.

2. Axis mark

Single-line text, e.g. "1", "1/2" etc.

Explanations

1. Axis mark shall be located (wholly or partially) inside one of two conventional zones adjacent to beginning or end of axis (fig. 1).

2. Each plan shall have at least one axis for X.

3. Each section (section view) shall have at least one of axes: either axis for X or axis for Y.

Fig. 1. Axis for X

2.4. IO Coordination axis for marking Y axis

Layer name:

"Axis for Y"

IO elements:

1. Axis

Horizontal line.

2. Axis mark

Single-line text, e.g. "A", "A1" etc.

Explanations

1. Mark location is similar to axis for X.

2. Each plan shall have at least one axis for Y.

3. Each section view shall have at least one of axes: either axis for X or axis for Y.

2.5. IO Coordination axis for marking Z axis

Layer name:

"Axis for Z"

IO elements:

1. Axis

Three-segment unclosed polyline consisting of axis itself

and callout lines.

2. Axis mark

Single-line text, e.g. "-3.000", "0.000" etc.

Explanations

1. Drawing of axis for Z shall start from a base point (fig. 2).

2. Axis mark shall be located (wholly or partially) inside a conventional zone around the horizontal line of callout (fig. 2).

3. All mirror variants of axis drawing are allowable.

4. Each section view shall have at least one axis for Z.

Fig. 2. Axis for Z

2.6. IO Section line

Layer name:

"Section lines"

IO elements:

1. Line

Three-segment unclosed polyline.

2. Section line mark

Single-line text, e.g. "1", "like 1" etc.

Significant information in presented text is the number 1.

Explanations

1. Section line mark shall be located (wholly or partially) inside one of two conventional zones adjacent to beginning or end of line (fig. 3).

2. Section line on plan has its corresponding section view on the drawing. E.g., section line with the mark "1" assumes that the drawing has section with the title "Section 1-1".

3. Section line has a zone of capture hereinafter referred to as a section line area. Section line area depth is taken equal to 1500 mm.

Fig. 3. Section line

2.7. IO Local section line

Layer name:

"Cutting lines"

Explanations

1. Local section line is similar to common section line but has the following differences:

- local section line is bound to a specific structural element;

- section view corresponding to local section line presents the cross-section of element, i.e. image on a plane perpendicular to structural element longitudinal axis;

- on the section view corresponding to local section line the coordination axes are not shown.

2. In the current version of interpreter the local section line can be shown only for rafters (see below).

2.8. IO Wall

Layer name:

"Walls"

IO elements

Wall on plan is shown by one- or multi- segment multiline that represents the intersection of wall with a horizontal plane.

Wall on section is shown by a vertical multiline that represents the intersection of wall with a vertical plane.

Explanations

1. Wall is a plane, rectangular, vertically oriented structure.

2. Each segment of multiline, representing on plan the wall with broken outline, is interpreted as a separate wall.

3. Binding of projections.

Conclusion of the fact that wall projection on plan and wall projection on section are projections of the same physical wall is confirmed by two conditions:

- wall projections on plan and section shall belong to the same story;

- affixments of limiting points on section line and limiting points on section view shall correspond to each other (fig. 4).

4. On section the belonging of wall to story is defined by the condition: wall bottom elevation shall be within the altitudinal range of story (fig. 5).

Fig. 4. Binding of wall projections

Fig. 5. Belonging of wall to story on the section

(walls 1 and 2 belong to story N)

Modeling per sample

If there are several walls on the plan, which are located on the same elevation and have identical height, highlight them with the same color. It is enough to draw a section line through one of the group walls. All group walls get the same height coordinates as the wall which is crossed by a section line.

Modeling by default

Lower edge of the walls not presented on sections and not belonging to any group is located by default at story elevation; height of these walls equals story height. The exception are walls of the building uppermost story; by default their height is taken equal to one meter.

Default modeling (this relates not only to walls but also other objects) is unreliable and shall be avoided whenever possible.

2.9. IO Front wall

Layer name:

"Front walls"

IO elements

On plan the front wall is shown by multiline indicating that the wall is cut by horizontal plane.

On section front wall is shown by polyline indicating the wall frontal view.

Explanations

1. Front wall is a plane, vertically oriented structure with arbitrary outline.

2. Binding of projections.

Conclusion of the fact that front wall projection on plan and front wall projection on section are projections of the same physical front wall is confirmed by three conditions:

- front wall projections on plan and section shall belong to the same story;

- front wall projection on plan shall be (wholly or partially) within section line area (fig. 6);

- affixments of limiting points on section line and limiting points on section view shall correspond to each other (fig. 6).

3. On section the belonging of front wall to story is defined as for usual wall (fig.5).

Fig. 6. Binding of front wall projections

Modeling per sample

Highlight on the plan a group of front walls located at the same elevation and having similar frontal view (including openings, see below) and thickness, with the same color. It is enough to draw a section line near one of the group walls. All group walls will get the same elevation and frontal view as the wall located within the section line area.

Modeling by default

Lower edge of the front walls not presented on sections and not belonging to any group is located by default at story elevation; height of these walls equals story height.

2.10. IO Front wall opening

Layer name:

"Front wall gaps"

IO elements

On section front wall opening is shown by polyline indicating its contour.

Explanations

Front wall opening is used for modeling through gaps of arbitrary outline in walls (fig. 7).

Fig. 7. Front wall openings

2.11. IO Slab

Layer name:

"Slabs"

IO elements

On plan the slab is shown by closed polyline indicating the slab outer contour.

On section slab is shown by horizontal multiline indicating the transversal cutting of slab by vertical plane.

Explanations

1. Slab is as flat, horizontally oriented structure of arbitrary outline.

2. Binding of projections.

Conclusion of the fact that slab projection on plan and slab projection on section are projections of the same physical slab is confirmed by two conditions:

- slab projections on plan and section shall belong to the same story;

- affixments of limiting points on section line and limiting points on section view shall correspond to each other (fig. 8).

3. On section the belonging of slab to story is defined by the condition: slab top elevation shall be within the altitudinal range of story (fig. 9).

Fig. 8. Binding of slab projections

Fig. 9. Belonging of slab to story on section

(slabs 1 and 2 belong to story N)

Modeling per sample

Highlight on the plan a group of slabs located at the same elevation, which have similar thickness, with the same color. It is enough to draw a section line through one of the group slabs. All group slabs will get the same height coordinates as the slab which is crossed by a section line.

Modeling by default

Upper edge of the slabs not presented on sections and not belonging to any group is located by default at story elevation; these slabs are 300 mm thick.

2.12. IO Slab opening

Layer name:

"Slab gaps"

IO elements

Slab opening on plan is shown by polyline indicating its contour.

Explanations

Slab opening is used for modeling through gaps of arbitrary outline in slabs (fig. 10).

Fig. 10. Slab openings

2.13. IO Ramp

Layer name:

"Ramps"

IO elements

On plan the ramp is shown by closed polyline indicating the ramp outer contour.

On section the ramp is shown by sloping multiline indicating the cutting of ramp by vertical plane.

Explanations

1. Generally, ramp is a sloping slab with a "base edge" (edge parallel to horizontal plane). When drawing ramp on plan the base edge is added first. Section line shall be drawn on ramp so that it crosses base edge.

2. Binding of projections.

Conclusion of the fact that ramp projection on plan and ramp projection on section are projections of the same physical ramp is confirmed by two conditions:

- ramp projections on plan and section shall belong to the same story;

- affixments of limiting points on section line and limiting points on section view shall correspond to each other (fig. 11).

3. On section the belonging of ramp to story is defined by the condition: elevation of the ramp axial line upper node shall be within the altitudinal range of story (fig. 12).

Fig. 11. Binding of ramp projections

Fig. 12. Belonging of ramp to story on section

(ramp 1 belongs to story N, ramp 2 belongs to story N+1)

Modeling per sample

Highlight a group of ramps on plan,

- having base edges located at the same elevation,

- having similar thickness and angle of slope to horizontal plane, with the same color. It is enough to draw a section line through one of the group ramps. All group ramps will get the same height coordinates as the ramp which is crossed by a section line.

Modeling by default

The ramps not presented on sections and not belonging to any group are modeled by default as the slabs, i.e. with upper edge at story elevation and 300 mm thick.

2.14. IO Column

Layer name:

"Columns"

IO elements

On plan the column is shown by closed polyline indicating that it is cut by horizontal plane.

On section the column is shown by vertical multiline indicating the overall projection of column on a section plane.

Explanations

1. Column is a vertically oriented structure of arbitrary section.

2. Binding of projections.

Conclusion of the fact that column projection on plan and column projection on section are projections of the same physical column is confirmed by three conditions:

- column projections on plan and section shall belong to the same story;

- column projection on plan shall be (wholly or partially) within section line area (fig. 13);

- affixments of limiting points on section line and limiting points on section view shall correspond to each other (fig. 13).

3. On section the belonging of column to story is defined as for the wall (fig. 5).

Fig. 13. Binding of column projections

Modeling per sample

Highlight on the plan a group of columns located at the same elevation and having similar height, with the same color. It is enough to draw a section line near one of the group columns. All group columns will get the same height coordinates as the column located within the section line area.

Modeling by default

Base of the columns not presented on sections and not belonging to any group is located by default at story elevation; height of these columns equals story height. The exception are columns of the building uppermost story; by default their height is taken equal to one meter.

2.15. IO Column opening

Layer name:

"Column gaps"

IO elements

Column opening on plan is shown by polyline indicating its contour.

Explanations

Column opening is used during modeling of columns with closed profile section (fig. 14).

Fig. 14. Column opening

2.16. IO Beam

Layer name:

"Beams"

IO elements

On plan the beam is shown by multiline indicating the overall contour of beam.

On section the beam is shown by polyline indicating the cross-section of beam by vertical plane.

Explanations

1. Beam is a horizontally oriented structure of arbitrary section.

2. Binding of projections.

Conclusion of the fact that beam projection on plan and beam projection on section are projections of the same physical beam is confirmed by two conditions:

- beam projections on plan and section shall belong to the same story;

- affixments of limiting points on section line and limiting points on section view shall correspond to each other (fig. 15).

3. On section the belonging of beam to story is defined as for the slab (fig. 9).

Fig. 15. Binding of beam projections

Modeling per sample

Highlight on the plan a group of beams located at the same elevation and having similar section, with the same color. It is enough to draw a section line through one of the group beams. All group beams will get the same height coordinates and cross-section as the beam which is crossed by section line.

Modeling by default

The upper edge of the beams not presented on sections and not belonging to any group is located by default at story elevation; these beams are of rectangular section B x H, where B – section width, taken equal to width of beam projection on plan; H – section height, by default taken equal to 300 mm.

2.17. IO Rafter

Layer name:

"Rafters"

IO elements

On plan the rafter is shown by multiline indicating the overall contour of rafter.

On section, corresponding to section line, the rafter is shown by multiline indicating the overall contour of rafter.

On section, corresponding to local section line, the rafter is shown by polyline indicating the cross-section of rafter.

Explanations

1. Generally, rafter is a sloping beam of arbitrary section having a base node. The base node of rafter is node from which drawing of rafter on plan starts (first node of multiline).

2. Binding of projections.

Conclusion of the fact that rafter projection on plan and rafter projection on section are projections of the same physical rafter is confirmed by three conditions:

- rafter projections on plan and section shall belong to the same story;

- rafter projection on plan shall be (wholly or partially) within section line area (fig. 16);

- affixments of limiting points on section line and limiting points on section view shall correspond to each other (fig. 16).

3. On section the belonging of rafter to story is defined as for the ramp (fig.12).

4. Rafter cross-section is represented in scale 10:1 (10 units of drawing = 1 mm).

Fig. 16. Binding of rafter projections.

Rafter cross-section.

Rafter modeling per sample

As a sample you can use rafter with two projections – common section view and cross-section view, set by common section and local section lines correspondingly.

Each rafter matching in color the sample rafter will get missing data (base node coordinates, angle of inclination to horizontal plane and cross-section) according to the following rules:

Case 1: common section view of rafter is available; cross-section view of rafter is unavailable.

Rafter will get height coordinates according to its common section view and the same cross-section as the sample rafter.

Case 2: common section view of rafter is unavailable; cross-section view of rafter is available.

Rafter will get height coordinates according to section view of the sample rafter and cross-section, set by its local section line.

Case 3: common section view of rafter is unavailable; cross-section view of rafter is unavailable.

Rafter will get height coordinates and cross-section according to common section view and cross-section of the sample.

Rafter modeling by default

Rafter not matching in color any sample rafter will get missing data (base node coordinates, angle of inclination to horizontal plane and cross-section) according to the following rules:

Case 1: common section view of rafter is available; cross-section view of rafter is unavailable.

Rafter will get height coordinates according to its common section view.

Rafter will get rectangular cross-section with width which equals width of rafter projection on plan, and height which equals rafter thickness on common section view.

Case 2: common section view of rafter is unavailable; cross-section view of rafter is available.

Rafter will be presented as a horizontal beam of set cross-section with upper edge elevation at the level of story elevation.

Case 3: common section view of rafter is unavailable; cross-section view of rafter is unavailable.

Rafter will be presented as a horizontal beam with upper edge elevation at the level of story elevation.

Rafter will get rectangular cross-section with width which equals width of rafter projection on plan, and height which equals 300 mm by default.

3. Story multiplication

The function of typical stories multiplication is not yet documented.

4. Step-by-step object detailing

This function is not yet documented.

5. Getting Started

"Studies" folder contains examples of drawings for modeling the separate structural elements – walls, columns, beams etc. Experiment with each structural element: introduce changes into drawing and see how this influences the model being created.

Having studied modeling of separate elements you can see how complex objects are modeled. The "Cottage" folder contains file with drawings of a small cottage. The “Garage” folder contains file with drawings of two-level underground parking.

6. Program installation and start

Supply package includes the following files and folders:

- 3d-i.vlx (program "3d-int" - interpreter of the construction

drawings);

- Manual.doc;

- model.dxf (model empty file);

- Studies (folder with source drawings to study and test program).

It is recommended to place the application: 3d-i.vlx in startup suite.

Drawing interpretation includes the following steps:

- load the source drawing, e.g. cott.dxf;

- in the command line enter: 3d-i (after command running the

application: 3d-c.lsp will be generated; this application

is located in the folder containing source drawing);

- load the model blank drawing: model.dxf;

- load application: 3d-c.lsp;

- in the command line enter: 3d-c (3D model will be generated).

Copyright notice

Copyright 2013-2021, Alexander Yampolsky